TK11固件升级方式

TK11 Firmware Upgrade Instructions

1、机器先关机，再按住“PTT+开机”进入下载模式

1. Turn off the radio first, then press and hold "PTT + Power Knob" at the same time to enter download mode.

2、插入写频线

2. Connect the programming cable.

3、在写频软件上选择正确的端口，再点击“固件更新”选择固件进行更新

3. In the programming software, select the correct COM port, then click "Firmware Update" and choose the firmware file to start the update.

4、升级成功后，机器重新开机即可

4. Once the upgrade is successful, restart the radio.